EnerTrack ESP32 Microcontroller Pin Map

Complete GPIO assignment for the current EnerTrack smart plug firmware

Completeness scope: This pin map is complete for every GPIO assigned by the current EnerTrack firmware configuration. It is not a full generic ESP32 board pinout. Hardware identity was confirmed from the connected board on COM11 using esptool.

# Board And Firmware Profile

| Item | Current Project / Detected Value |
| --- | --- |
| Chip type | ESP32-D0WD-V3 |
| Chip revision | v3.1 |
| Board class | ESP32 Dev Module / ESP32-WROOM-32 class, common 38-pin development board |
| USB-to-UART bridge | Silicon Labs CP210x USB to UART Bridge |
| Detected COM port | COM11 |
| CPU / cores | Dual core, up to 240 MHz |
| Wireless features | Wi-Fi, Bluetooth, dual core + low-power core |
| Crystal frequency | 40 MHz |
| Detected flash size | 4 MB |
| Flash voltage | 3.3 V |
| MAC address | 00:4B:12:3B:C6:54 |
| Arduino IDE board | ESP32 Dev Module |
| Arduino IDE partition scheme | Huge APP (3MB No OTA / 1MB SPIFFS) |
| Firmware build target | esp32:esp32:esp32:PartitionScheme=huge_app |
| Firmware folder | firmware/enertrack-esp32 |
| Logic level | 3.3 V GPIO logic |
| Document date | June 1, 2026 |

# Complete GPIO Assignment

| GPIO | Firmware Role | Direction | Connected Part | Notes |
| --- | --- | --- | --- | --- |
| GPIO2 | Pairing/status LED | Output | Onboard/status LED | Active high. On many ESP32 DevKit boards this is the onboard LED. |
| GPIO4 | DS1302 RTC RST | Output | DS1302 RTC module | RTC reset/chip-enable line. |
| GPIO5 | microSD CS | Output | microSD card adapter | SPI chip select. Keep inactive/high during boot. |
| GPIO16 | PZEM RX | UART2 input | PZEM-004T TX | PZEM TX connects to ESP32 GPIO16. |
| GPIO17 | PZEM TX | UART2 output | PZEM-004T RX | ESP32 GPIO17 connects to PZEM RX. |
| GPIO18 | microSD SCK | SPI output | microSD card adapter | SPI clock. |
| GPIO19 | microSD MISO | SPI input | microSD card adapter | SPI data from SD card to ESP32. |
| GPIO21 | DS1302 RTC CLK | Output | DS1302 RTC module | RTC clock line. |
| GPIO22 | DS1302 RTC IO | Bidirectional | DS1302 RTC module | RTC data line. |
| GPIO23 | microSD MOSI | SPI output | microSD card adapter | SPI data from ESP32 to SD card. |
| GPIO25 | Manual relay button | Input pull-up | Push button to GND | Active low. Debounce: 60 ms. |
| GPIO26 | Relay control | Output | Relay module IN | Active high relay control. |
| GPIO27 | Pairing reset button | Input pull-up | Push button to GND | Active low. Hold 3 seconds to clear pairing. |
| GPIO32 | MP3 UART RX | UART1 input | DFPlayer/MP3 module TX | Receives serial data from MP3 module. |
| GPIO33 | MP3 UART TX | UART1 output | DFPlayer/MP3 module RX | Sends serial commands to MP3 module. |

# Module Wiring Summary

| Module | ESP32 Pins | Power / Reference | Project Notes |
| --- | --- | --- | --- |
| PZEM-004T-100A | GPIO16 RX, GPIO17 TX | PZEM VCC to 5 V/VIN, GND common | Measures voltage, current, watts, kWh, and power factor. Observe AC mains safety. |
| Relay module | GPIO26 | Relay VCC 5 V, GND common | Active high in firmware. Relay input should change 0 V/3.3 V from the ESP32. |
| Manual relay button | GPIO25 and GND | Uses ESP32 internal pull-up | Pressing the button toggles the relay locally and updates cloud state when available. |
| Pairing reset button | GPIO27 and GND | Uses ESP32 internal pull-up | Clears saved pairing and resets PZEM energy when enabled. |
| microSD module | GPIO18, 19, 23, 5 | 5 V adapter module, GND common | Stores offline logs. SPI frequency configured at 1 MHz. |
| DS1302 RTC | GPIO21, 22, 4 | VCC per module, GND common | Provides offline timestamps. Sync interval: 6 hours. |
| DFPlayer/MP3 module | GPIO32 RX, GPIO33 TX | VCC per module, GND common | 9600 baud. Voice tracks use /MP3/0001.mp3 through /MP3/0036.mp3. |

# Firmware Timing And Electrical Limits

| Setting | Value |
| --- | --- |
| PZEM reading interval | 6 seconds |
| Firestore reading archive interval | 30 seconds |
| Relay/protection command polling | 1 second |
| Pairing reset hold time | 3 seconds |
| Pairing reboot delay | 15 seconds |
| RTC sync interval | 6 hours |
| Protection limits | 5.0 A and 1100 W |
| PZEM valid voltage range | 80 V to 300 V |
| PZEM maximum valid current/power sanity range | 100 A / 25000 W |
| microSD SPI frequency | 1 MHz |
| MP3 UART baud / volume | 9600 baud / volume 30 |
| MP3 voice queue | 12 tracks max, 2.6 seconds gap |